Variable frequency signal generation

Embedded Systems

Intermediate Exam 1

Done by: EKSfmu-16 gr. st. Arūnas Butkus

Checked by: doc. dr. Andrius Ušinskas

Work goal: using DE0 development board generate periodical signal showing frequency using HEX displays and allowing to change said frequency with switches.

Algorithms used in tasks.

Fig. 1. “Algorithm” for signal generation.

Results analysis.

Fig. 2. Signal generation without any frequency division.

All in all it is simple operation. First on each clock rising edge a counter is increased and that value is tested against a threshold value x and, if counter is greater than x, then it is reset and another counter, y, is increased. Threshold value x is calculated from the switches – switches representing a binary number. 10 switches give x value ranging from 0 to 1023. In practice, x is on or off cycle count, since the second counter is used to determine the output state – output high when counter even, output low when counter odd.  This means that it is effectively a frequency divider. In same cycle frequency for display is also determined, by a simple equation. HEX displays are directly tied to that frequency value. Examples of operation are shown in Figure 3 and can also be viewed on YouTube at https://youtu.be/Wulkias8waE

Fig. 3. Few examples of signal generation using the divider.

Conclusions.

Well, it works and is quite precise. The 4 HEX displays can’t provide much precision when indicating target frequency in some cases, but does point to the right direction. By the way the signal is a square wave with maximum frequency of 25 MHz. And in this particular setup minimum frequency is 48 kHz.

Source code.

001 library ieee;

002 use ieee.std_logic_1164.all;

003 use ieee.numeric_std.all;

004

005 entity IESigGen is

006 	Port (

007 		CLOCK_50 : IN STD_LOGIC;

008 		SW : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

009 		KEY : IN STD_LOGIC_VECTOR(2 DOWNTO 0);

010 		HEX0 : OUT STD_LOGIC_VECTOR(7 DOWNTO 0);

011 		HEX1 : OUT STD_LOGIC_VECTOR(7 DOWNTO 0);

012 		HEX2 : OUT STD_LOGIC_VECTOR(7 DOWNTO 0);

013 		HEX3 : OUT STD_LOGIC_VECTOR(7 DOWNTO 0);

014 		LEDG : OUT STD_LOGIC_VECTOR(9 downto 0);

015 		GPIO_0 : OUT STD_LOGIC_VECTOR(2 DOWNTO 0)

016 	);

017 end IESigGen;

018

019 architecture Whatever of IESigGen is

020

021 signal timer_count   : INTEGER := 0;

022 signal counter    	 : INTEGER := 0;

023 SIGNAL FREQUENCY     : INTEGER := 0;

024 signal INTERVAL      : INTEGER := 0;

025

026 begin

027

028 	WITH FREQUENCY mod 10 SELECT HEX0 <=

029 		"11000000" WHEN 0,

030 		"11111001" WHEN 1,

031 		"10100100" WHEN 2,

032 		"10110000" WHEN 3,

033 		"10011001" WHEN 4,

034 		"10010010" WHEN 5,

035 		"10000010" WHEN 6,

036 		"11111000" WHEN 7,

037 		"10000000" WHEN 8,

038 		"10010000" WHEN 9,

039 		"01111111" WHEN OTHERS;

040

041 	WITH (FREQUENCY / 10) mod 10 SELECT HEX1 <=

042 		"11000000" WHEN 0,

043 		"11111001" WHEN 1,

044 		"10100100" WHEN 2,

045 		"10110000" WHEN 3,

046 		"10011001" WHEN 4,

047 		"10010010" WHEN 5,

048 		"10000010" WHEN 6,

049 		"11111000" WHEN 7,

050 		"10000000" WHEN 8,

051 		"10010000" WHEN 9,

052 		"01111111" WHEN OTHERS;

053

054 	WITH (FREQUENCY / 100) mod 10 SELECT HEX2 <=

055 		"01000000" WHEN 0,

056 		"01111001" WHEN 1,

057 		"00100100" WHEN 2,

058 		"00110000" WHEN 3,

059 		"00011001" WHEN 4,

060 		"00010010" WHEN 5,

061 		"00000010" WHEN 6,

062 		"01111000" WHEN 7,

063 		"00000000" WHEN 8,

064 		"00010000" WHEN 9,

065 		"01111111" WHEN OTHERS;

066

067 	WITH (FREQUENCY / 1000) mod 10 SELECT HEX3 <=

068 		"11000000" WHEN 0,

069 		"11111001" WHEN 1,

070 		"10100100" WHEN 2,

071 		"10110000" WHEN 3,

072 		"10011001" WHEN 4,

073 		"10010010" WHEN 5,

074 		"10000010" WHEN 6,

075 		"11111000" WHEN 7,

076 		"10000000" WHEN 8,

077 		"10010000" WHEN 9,

078 		"01111111" WHEN OTHERS;

079

080 	WITH COUNTER MOD 2 SELECT GPIO_0 <=

081 		"001" WHEN 0,

082 		"000" WHEN OTHERS;

083

084 	process(CLOCK_50)

085 	begin

086 		if rising_edge(CLOCK_50) then

087 			timer_count <= timer_count + 1;

088 			if (timer_count >= INTERVAL) then

089 				counter <= counter + 1;

090 				timer_count <= 0;

091 			end if;

092

093 			INTERVAL <= TO_INTEGER(UNSIGNED(SW));

094 			FREQUENCY <= 5000/((INTERVAL+1)*2);

095

096 		end if;

097 	end process;

098

099 end Whatever;

Electrical schemes: